Lab 4: Second Timer using an Interrupt Controller and 7-Segment Display

## Prerequisites

Prior to starting this guide make sure to perform Lab 3 to understand how to use Vivado block diagrams to connect new IP to the MicroBlaze soft processor.

Part 1: Synthesizing Block Diagram in Vivado

1.) Create a new Xilinx Vivado project just as you did for Lab 1, Part 2: Getting Started in MicroBlaze.  Call the Project “lab4.xpr”.  Then, create a new Block Diagram and name it “lab4a”.  As a reminder, the steps are:

1.1: From the start page, select the Create New Project button to start the New Project Wizard.

#### 1.2: Click Next to continue to the first step.

#### 1.3: Set the name of the project to “lab4”. Vivado will use this name when generating its folder structure.

Important: Do NOT use spaces in your project name or location path.

1.4 Choose Project Type as RTL Project. Leave the “Do not specify sources box” cleared and click Next.

1.5 Click next until you can select Boards. From the filter options make required selections for Vendor, Display Name and Board Revision. “Basys 3” should be displayed in the selection list. A mismatch in selecting the correct board name will cause errors. Choose Basys 3 and click next.

1.6 A summary of the new project design sources and target device is displayed. Click Finish.

1.7 On the left in the Flow Navigator, select Create Block Design under the IP Integrator, and name your design “lab4a”.

2.) In the Sources window, click the Board tab.  Click and drag the following Basys3-related IP blocks into your empty block diagram:

2.1. Clocks  System Clock.  This places your sys_clock input port and also inserts a new Clocking Wizard.  Double-click the Clocking Wizard block and select the Output Clocks tab to ensure that the Output Clock (clk_out1) is set to an Output Freq of 100 MHz and Active High Reset Type.

2.2 GPIO  4 Push Buttons.  This also inserts your 1st AXI GPIO (0) block.

2.3 GPIO  16 LEDs.  Drag this on top of the existing block named axi_gpio_0.  It connects the port to a new block output named GPIO2.

2.4 GPIO  7 segment display – Anodes.  This also inserts a 2nd AXI GPIO (1) block.

2.5 GPIO  7 segment display – Segments. Drag this on top of the existing block named axi_gpio_1.  It connects the port to a new block output named GPIO2.

2.6 UART  USB UART.  This places your usb_uart output port and also inserts a new AXI Uartlite block.

3.) In the Diagram window, select the Add IP icon, or right-click and select “Add IP…” from the context menu.  In the search bar, enter the following 2 search terms, which inserts the following IP blocks:

3.1 “microblaze”  MicroBlaze block

3.2 “timer”  AXI Timer block

4.) Next to where it displays “Designer Assistance available”, select Run Block Automation and a customization assistant window will open with default settings.  Select the following Options:

| Local Memory | 128 KB |
| --- | --- |
| Local Memory ECC / Cache Configuration | None |
| Debug Module | Debug Only |
| Peripheral AXI Port | Enabled |
| Interrupt Controller | Checked |
| Clock Connection | /clk_wiz_0/clk_out1 (100 MHz) |

These settings automatically insert new MicroBlaze Debug Module (MDM), Processor System Reset, microblaze_0_local_memory, AXI Interconnect, Concat, and AXI Interrupt Controller IP blocks.  Ensure that the AXI Interrupt Controller’s only output port, “interrupt”, is connected to MicroBlaze’s 1st input port, “INTERRUPT”.

5.) Next to where it displays “Designer Assistance available”, select Run Connection Automation, and a new window will open.  Select the checkbox next to “All Automation”, and click OK.  You should see that new connections have been drawn between the IP blocks.

6.) Manually draw a connection from the AXI Timer’s “interrupt” output port to the Concat block’s In0 input port.  To do so, click the line next to AXI Timer/interrupt and drag the mouse to the line next to Concat/In0[0:0].  To show you that the connection is valid, Vivado will (1) put a green checkmark next to Concat/In0[0:0], (2) draw a thick straight black line between the two ports, and (3) display a tooltip that says “Connect from ‘interrupt’ port to ‘In0’ port”.  In the same manner, manually draw a connection between axi_gpio_1/ip2intc_irpt and Concat/In1.

7.) Select the Regenerate Layout () button to clean up the block diagram.  The completed diagram should appear like the following:

Next, select the Validate Design () button to check for errors.

8.) Select the “Sources” tab.  Below “Design Sources”, select your block diagram “lab4a” given by the  icon, right-click it, and select “Create HDL Wrapper”. In the “Create HDL Wrapper” window that appears, select “Let Vivado manage wrapper and auto-update”, and then click OK.

9.) In the Flow Navigator pane on the left side, select  to run Synthesis.  Select  to Run Implementation.  Select  to generate a bitstream.

10.) On the main toolbar, click File and select Export→Export Hardware. Check the box to Include Bitstream and click OK. This will export the hardware design with system wrapper for the Software Development Tool - Vivado SDK. 

A new file directory will be created under lab4.SDK similar to the Vivado hardware design project name. Two other files, .sysdef and .hdf are also created. This step essentially creates a new SDK Workspace.

In the Vivado top menu, click File and then Launch SDK. Leave both dropdown menus as their default Local to Project and click OK. This will open Xilinx SDK and import your hardware.

### Part 2: Determining Timer Reset Value and 7-Segment Display binary codes

### While your bitstream is being generated by Vivado in Part 1, you should determine the ideal reset value for the timer and the values to assign to the 7-segment display cathodes for each possible number.

### 2A: First, decide upon a Reset Value to assign to the AXI Timer.

### The AXI Timer maintains a 32-bit (4-byte) value that increments by 1 upon each clock cycle.

### Its input clock was provided by the Clocking Wizard, which you had verified in Step 2.1 was 100 MHz (100×106=108 Hz); thus, you can deduce that it increments every 10 ns (10×10–9=10–8 s).

### The timer throws an interrupt when it overflows, exceeding its maximum possible value of 232–1 = 0xFFFFFFFF and must be reset to a “Reset Value”.  The choice of the Reset Value is left to the programmer.

### Determine the 8-digit hexadecimal number to which the Reset Value should be set in order to throw an interrupt every 1 ms=10–3 s, and write it down here.  Note that the Windows Calculator program has a “Programmer” mode of operation that can easily convert between binary, decimal, and hexadecimal numbers.

### 2B: Next, the 7-segment display should be configured enable the individual cathodes that need to be lit in order to display each number.  The letters shown in the figure above (from the Basys 3 Reference Manual here: https://reference.digilentinc.com/basys3/refmanual) correspond to the 8 bits of the seven_seg_led_disp interface on GPIO #1 channel #2, where A is the least significant bit (LSB, bit 0), B is bit 1, C is bit 2, D is bit 3, E is bit 4, F is bit 5, G is bit 6, and DP is the most significant bit (MSB, bit 7).

### Fill in the lookup table below such that for each of 10 possible numeric digits, you write the corresponding binary sequence for the cathodes (from MSB to LSB) and the associated decimal or hex value.  
Note: Both the anodes and cathodes should have a ‘0’ bit written to light up the corresponding segment; however, for this exercise, you may assume that the binary sequence may be ‘1’ for a lit cathode and ‘0’ for an unlit cathode, and the value will be simply negated later (with the C binary ~ operator) in the ISR.

| ### Digit | ### Cathodes Binary Sequence: 7 6 5 4 3 2 1 0 | ### Hex / Decimal value |
| --- | --- | --- |
| ### 0 |  |  |
| ### 1 |  |  |
| ### 2 |  |  |
| ### 3 |  |  |
| ### 4 |  |  |
| ### 5 |  |  |
| ### 6 |  |  |
| ### 7 |  |  |
| ### 8 |  |  |
| ### 9 |  |  |

To verify your responses, you may call over the instructor and get checked off for Part 2 at this point.

### Part 3: Creating New Application Project in SDK

In the SDK top menu, click the New dropdown arrow and select Xilinx→Application Project.
Give your project the name “lab4b” (with no empty spaces), ensure in the Hardware Platform dropdown box that the most recently exported Lab 4 HW platform is selected, and click Next.

On the Templates page, select “Hello World” and click Finish.

11.) In the Project Explorer, right-click helloworld.c under the src folder and rename it to “lab4c.c” or similar.
On line 47, ensure that all of the following #include statements are entered.

/* Include Files */

#include <stdio.h>	/* Contains the standard input and output functions */

#include "platform.h"	/* Defines init and tidy up routines for MicroBlaze */

#include "xil_printf.h"

#include "xparameters.h" /* Contains address & configurations system needs */

#include "xtmrctr.h"

#include "xgpio.h"	/* General Purpose I/O */

#include "xstatus.h"	/* Contains the status definitions used */

#include "xintc.h"	/* Contains API for using the AI interrupt controller */

#include "xil_exception.h"	/* Contains the API for MicroBlaze exceptions */

12.) On line 58, beneath the #include statements, ensure the following #define statements are present.

/* Definitions */

#define GPIO_DEVICE0_ID XPAR_AXI_GPIO_0_DEVICE_ID /* GPIO dev for LEDs & btns */

#define GPIO_DEVICE1_ID XPAR_AXI_GPIO_1_DEVICE_ID /* GPIO dev for 7-seg disp */

#define LED 0x0000		/* Initial LED value */

#define CAT 0x00			/* Initial 7-segment display cathode value */

#define DELAY_7SEG 100		/* Delay between anodes of 7-segment display */

#define DELAY_DISP 1000		/* Delay between updates to displays */

#define BTN_CHANNEL 1		/* GPIO port for pushbuttons */

#define LED_CHANNEL 2		/* GPIO port for LEDs */

#define ANODE_CHANNEL 1		/* GPIO port for 7-segment display anodes */

#define CATHODE_CHANNEL 2	/* GPIO port for 7-segment display cathodes */

#define printf xil_printf 	/* A smaller, optimized printf */

#define TMRCTR_DEVICE_ID XPAR_TMRCTR_0_DEVICE_ID

#define TMRCTR_INTERRUPT_ID XPAR_INTC_0_TMRCTR_0_VEC_ID

#define INTC_DEVICE_ID XPAR_INTC_0_DEVICE_ID

#define TIMER_CNTR_0 0

#define INTC XIntc

#define INTC_HANDLER XIntc_InterruptHandler

#define RESET_VALUE 0x????????

On line 76, replace the expression “0x????????” with the 8-digit hexadecimal number you calculated in Part 2A.  Don’t call the instructor over to ask why this line is causing build errors. 🙄

13.) On line 78, beneath the #define statements, add the following Global variable declarations:

/* Global Variables */

XGpio Gpio0;			/* GPIO Device driver instance 1 */

XGpio Gpio1;			/* GPIO Device driver instance 2 */

INTC InterruptController; 	/* The instance of the Interrupt Controller */

XTmrCtr TimerCounterInst;	/* The instance of the Timer Counter */

volatile int numMillisec = 0;

volatile int whichDigit = 0;

volatile int led = LED;		/* Hold current LED value. */

volatile int cath[4]; 		/* Hold current cathode value. */

14.) On line 88, beneath the global variable declarations, add the following function declarations:

/* Function Declarations */

void lookup7Seg(void);

int Lab4Timer(INTC* IntcInstancePtr,XTmrCtr* TmrCtrInstancePtr,

u16 DeviceId,u16 IntrId,u8 TmrCtrNumber);

void TimerCounterHandler(void *CallBackRef, u8 TmrCtrNumber);

static int TmrCtrSetupIntrSystem(INTC* IntcInstancePtr,

XTmrCtr* TmrCtrInstancePtr,u16 DeviceId,u16 IntrId,u8 TmrCtrNumber);

void TmrCtrDisableIntr(INTC* IntcInstancePtr, u16 IntrId);

After the main() function, you will have to define every one of these functions below.

15.) On line 97, replace the existing main() function contents with the following code:

int main() {

init_platform();

int Status;

Status = Lab4Timer(&InterruptController, &TimerCounterInst,

TMRCTR_DEVICE_ID, TMRCTR_INTERRUPT_ID, TIMER_CNTR_0);

if (Status != XST_SUCCESS) {

xil_printf("Timer Interrupt Counter Failed!\r\n");

return XST_FAILURE;

}

return XST_SUCCESS;

}

16.) On line 110, after the main() function, add the lookup7Seg() function definition, which implements the lookup table that you had envisioned in Part 2B.

void lookup7Seg(void) {

int i, digit, num;

num = led;

for (i=0; i<4; i++) {

digit = num % 10;

if (digit==0) 	 { cath[i] =   0; } 	/* 0x3F = 0b00111111 */

else if (digit==1) { cath[i] =   0; }	/* 0x =0b00000000 */

else if (digit==2) { cath[i] =   0; }	/* 0x =0b00000000 */

else if (digit==3) { cath[i] =   0; } 	/* 0x =0b00000000 */

else if (digit==4) { cath[i] =   0; } 	/* 0x =0b00000000 */

else if (digit==5) { cath[i] =   0; } 	/* 0x =0b00000000 */

else if (digit==6) { cath[i] =   0; } 	/* 0x =0b00000000 */

else if (digit==7) { cath[i] =   0; } 	/* 0x =0b00000000 */

else if (digit==8) { cath[i] =   0; } 	/* 0x =0b00000000 */

else if (digit==9) { cath[i] =   0; } 	/* 0x =0b00000000 */

else 			 { cath[i] =   0; } 	/* 0x00=0b00000000 */

num /= 10;

}

return;

}

This function uses two global variables.  Its input, led, is the current count of tenths of seconds.  For each of the 4 digits on the 7-Segment display, it sets the corresponding cathode segments in the 4-element integer array, cath.  On each if/else if line, replace the 0 with the corresponding hex or decimal value that you calculated in Part 2B.  For debugging purposes, you may also replace the comments on each line with the corresponding binary sequence (e.g. 01010101) for that digit. Don’t call the instructor over to ask why your 7-segment display is blank if you haven’t updated this function yet. 🙄

17.) On line 131, after the lookup7Seg() function definition, define your Lab 4 Timer function as shown:

int Lab4Timer(INTC* IntcInstancePtr,XTmrCtr* TmrCtrInstancePtr,

u16 DeviceId,u16 IntrId,u8 TmrCtrNumber) {

int Status, Mode = 0;

uint8_t btn, last_btn=0x00; /* Hold current & last pushbutton values */

Status = XTmrCtr_Initialize(TmrCtrInstancePtr, DeviceId);

if (Status != XST_SUCCESS) { return XST_FAILURE; }

Status = XTmrCtr_SelfTest(TmrCtrInstancePtr, TmrCtrNumber);

if (Status != XST_SUCCESS) { return XST_FAILURE; }

Status = TmrCtrSetupIntrSystem(IntcInstancePtr,TmrCtrInstancePtr,DeviceId,

IntrId,TmrCtrNumber);

if (Status != XST_SUCCESS) { return XST_FAILURE; }

XTmrCtr_SetHandler(TmrCtrInstancePtr,TimerCounterHandler,

TmrCtrInstancePtr);

XTmrCtr_SetOptions(TmrCtrInstancePtr,TmrCtrNumber,

XTC_INT_MODE_OPTION | XTC_AUTO_RELOAD_OPTION);

XTmrCtr_SetResetValue(TmrCtrInstancePtr, TmrCtrNumber, RESET_VALUE);

XTmrCtr_Start(TmrCtrInstancePtr, TmrCtrNumber);

/* GPIO driver initialization */

Status = XGpio_Initialize(&Gpio0, GPIO_DEVICE0_ID);

Status = XGpio_Initialize(&Gpio1, GPIO_DEVICE1_ID);

if (Status != XST_SUCCESS) { return XST_FAILURE; }

/* Set the direction for the LEDs to output */

XGpio_SetDataDirection(&Gpio0, BTN_CHANNEL, 0xFF);

XGpio_SetDataDirection(&Gpio0, LED_CHANNEL, 0x00);

XGpio_SetDataDirection(&Gpio1, ANODE_CHANNEL, 0x00);

XGpio_SetDataDirection(&Gpio1, CATHODE_CHANNEL, 0x00);

xil_printf("Entering while(1).\r\n");

while (1) {

/* Do nothing in main loop initially except handle interrupts.*/

/* Later, replace these comments with your Part 4 algorithm. */

}

xil_printf("Exiting while(1).\r\n");

TmrCtrDisableIntr(IntcInstancePtr, DeviceId);

return XST_SUCCESS;

}

Note that this function initializes the timer, interrupt system, and GPIO devices.  For Part 4, you may replace the comments within the while (1) loop with some pushbutton-handling logic.

18.) After the Lab4Timer() function, define the TimerCounterHandler() ISR function.

void TimerCounterHandler(void *CallBackRef, u8 TmrCtrNumber) {

XTmrCtr *InstancePtr = (XTmrCtr *)CallBackRef;

if (XTmrCtr_IsExpired(InstancePtr, TmrCtrNumber)) {

if (numMillisec == 0) {

XGpio_DiscreteWrite(&Gpio0, LED_CHANNEL, led);

lookup7Seg();

led++;

}

numMillisec = (numMillisec+1) % 100;

if (whichDigit==0) {

XGpio_DiscreteWrite(&Gpio1,ANODE_CHANNEL,~(0x1));

XGpio_DiscreteWrite(&Gpio1,CATHODE_CHANNEL,~(cath[0]));

} else if (whichDigit==1) {

XGpio_DiscreteWrite(&Gpio1,ANODE_CHANNEL,~(0x2));

XGpio_DiscreteWrite(&Gpio1,CATHODE_CHANNEL,~(cath[1]|0x80));

} else if (whichDigit==2) {

XGpio_DiscreteWrite(&Gpio1,ANODE_CHANNEL,~(0x4));

XGpio_DiscreteWrite(&Gpio1,CATHODE_CHANNEL,~(cath[2]));

} else if (whichDigit==3) {

XGpio_DiscreteWrite(&Gpio1,ANODE_CHANNEL,~(0x8));

XGpio_DiscreteWrite(&Gpio1,CATHODE_CHANNEL,~(cath[3]));

}

whichDigit = (whichDigit+1) % 4;

}

}

In this function, the Interrupt Service Routine (ISR) is defined.  Since we expect the ISR to be called every 1ms, we can activate each of the anodes (representing each digit) one at a time.  Note that we negate each anode and cathode here to provide the input to the 7-segment display in an active low manner, and a binary OR on digit 1 with 0x80 is used to hold a decimal point before the last digit.  The digit is incremented and modulo 4 to ensure that each anode is updated every 4 ms.  Every 100 ms, the lookup7Seg() function is called to find the new cathode values for this time.

19.) After the TimerCounterHandler() function, define the Timer Setup and Disable functions.

static int TmrCtrSetupIntrSystem(INTC* IntcInstancePtr,

XTmrCtr* TmrCtrInstancePtr,u16 DeviceId,u16 IntrId,u8 TmrCtrNumber) {

int Status;

Status = XIntc_Initialize(IntcInstancePtr, INTC_DEVICE_ID);

if (Status != XST_SUCCESS) { return XST_FAILURE; }

Status = XIntc_Connect(IntcInstancePtr, IntrId,

(XInterruptHandler)XTmrCtr_InterruptHandler,

(void *)TmrCtrInstancePtr);

if (Status != XST_SUCCESS) { return XST_FAILURE; }

Status = XIntc_Start(IntcInstancePtr, XIN_REAL_MODE);

if (Status != XST_SUCCESS) { return XST_FAILURE; }

XIntc_Enable(IntcInstancePtr, IntrId); /* Enable interrupt for timer ctr */

Xil_ExceptionInit();			/* Initialize the exception table. */

Xil_ExceptionRegisterHandler(XIL_EXCEPTION_ID_INT,

(Xil_ExceptionHandler) INTC_HANDLER,IntcInstancePtr);

Xil_ExceptionEnable();			/* Enable non-critical exceptions. */

return XST_SUCCESS;

}

void TmrCtrDisableIntr(INTC* IntcInstancePtr, u16 IntrId) {

/* Disable the interrupt for the timer counter */

XIntc_Disable(IntcInstancePtr, IntrId);

return;

}

20.) Save the C file.  Program FPGA; on the top toolbar, click the Program FPGA button.

Start a TeraTerm window using the same settings as in Labs 1 and 3. Build and run the executable; click the Run As… button, select Launch on Hardware (System Debugger), and click OK.

Ensure that you can see the “Entering while(1)” terminal message, the LEDs cycling, and the 7-segment display updating with the appropriate numeric digits at the appropriate rates.  If your update rate is off, review Part 2A; if your numeric digits don’t display correctly, review your Part 2B response.

Once you’ve completed these steps, you may call over the instructor to mark you down for Part 3.

### Part 4: A First Step Towards Making a Stopwatch

For part 4, you will implement functionality for a start/stop button.  The timer display should stop and restart when the user presses the Right pushbutton.  When the Right button is pressed and the timer is presently counting, you should stop the timer (only once) at its present count.  When the Right button is pressed again while the timer is stopped, you should restart the timer (only once) where it last left off.

While you’re encouraged to implement this logic on your own, you are welcome to note the following hints:

- The “while(1)” loop in the Lab4Timer function is the easiest place to implement the start/stop button functionality.

- Recall from Lab 3 that the XGpio_DiscreteRead() function is used to get input data from a pushbutton connected through an AXI GPIO interface.

- Keeping a local integer variable, Mode, will help you to reconcile whether the Stopwatch is presently in a “Stopped” or a “Running” state.

- To ensure that you recognize the pushbutton is pressed only once, you should have your if statement look for “rising edges” in the pushbutton values.  That is, if you maintain a current btn value and a past last_btn value, you can acknowledge a button press only when btn is high (for the Right button) and last_btn was low.

- The XTmrCtr_Stop() function can be called to stop the timer at its present count. While its definition is listed in the xtmrctr.h include file for the generated board support package (lab4b_bsp), you may note that it takes the same inputs as the XTmrCtr_Start() and also gives no outputs.

- The XTmrCtr_Start() function can be called to resume the timer at its last saved count.  This function is already presently called in the Lab4Timer() function.

Note that the display does not continue to update if you halt the timer in this fashion.  Why is this?  What alternate design steps could you take to ensure that the 7-segment display refreshes the saved time value until the pushbutton is pressed again?

Once you’ve completed these steps, you may call over the instructor to mark you down for Part 4.

Submit  .c code for credit. Any unfinished sections that the instructor didn’t check can also be submitted.