NAME:  ______John feet_____________

We will be using the following components for this lab:

- Digilent Joystick Module

- Basys3 FPGA

- Xilinx SDK terminal

We will be using the terminal in the Xilinx SDK to display the X and Y coordinates received from the joystick.

Even though we are using SPI, we still need to set the console to work at the following protocol: 115200 8N1.

PART A – Connecting the Joystick

Refer to the Basys3 Connections document. If you notice, the SPI ports are the upper left block labeled JA. Each row represents a different clock speed used for the synchronous communication. Based on the Joystick documentation, the maximum speed is 1 MHz. So given that constraint, the top row of the block should be used. Luckily the pins line up, so no jumper wires are required.

PART B  - Swapping bytes from master device (Basys3) and slave device (PmodJSTK)

Using the given flowcharts for lab 5, write two separate subroutines and a main that will initialize communication with the joystick and then start exchanging data. The terminal should display the coordinates.

When following the flowchart, there are suggestions on the expected code in the process blocks. Use that to help you. Also avoid common pitfalls. Those are usually the following:

- Not configuring the SPI with the correct configuration values. Unlike other scenarios, you don’t have to mask here.

- Not sending dummy bytes. Make sure the value is exactly a byte.

- Incorrect wait condition. Know when to use IF and when to use WHILE

- No delay between bytes sent (documentation suggests 10 microsec)

- Shifting pointer manually in buffers. This is unnecessary.

- Not organizing high and low bytes properly.

- Setting up global values incorrectly (or overwriting them by making them local).

Print using the standard printf() function. Your coordinates should be displayed in HEX, (%x) as the number will be much smaller and easier to read. Based on the range given in the documentation of x and y (0-1024), your values should range between 0 and 1FF.  Rather than have your console print coordinates on a new line, instead use the syntax of ‘\r’. This will create a carriage return, bringing the cursor back to the beginning of the line and overwriting the previous value. It will make it much easier to read the number in real time this way.

FINISHED CODE:

void SPI_initialize();

void SPI_joystick_exchange(unsigned char switches);

// Global variables

unsigned char buttons;

unsigned short x_position, y_position;

int main(){

unsigned char switches;

SPI_initialize();

while (1){

switches = SWITCH_DATA;

switches &= 0x01;

SPI_joystick_exchange(switches);

printf("x_pos: 0x%03x y_pos: 0x%03x \n", x_position, y_position); //03 bc 0x3FF is max

LED_DATA = buttons;

}

}

void SPI_initialize(){

//SPICR = 0x086;

SPICR = MANUAL_SLAVE_SELECT + MASTER_MODE + SPI_ENABLED;//0x086

SPISSR = 1;

}

void SPI_joystick_exchange(unsigned char switches){

unsigned char xlow, xhigh, ylow, yhigh, status, dummy;

dummy = 0; //Can be anything

SPISSR = 0;

delay_us(15);

switches |= 0x80;

SPIDTR = switches;

delay_us(10);

for (int i=0; i<4; i++){ //send 4 dummy bites

SPIDTR = dummy;

delay_us(10);}

status = SPISR;

status &= SPI_TX_FIFO_EMPTY;

while (status != SPI_TX_FIFO_EMPTY){ //wait for data

status = SPISR;

status &= SPI_TX_FIFO_EMPTY;}

xlow = SPIDRR; //take 1st bite <-1st bite is now gone

xhigh = SPIDRR; //take 2nd <- 2nd bit is now gone

ylow = SPIDRR;

yhigh =  SPIDRR;

buttons = SPIDRR; //LMAO we dont need this (will be universal)

// "<<n" = "*2^n"

x_position = (xhigh<<8) + xlow; //shift xhigh left and the add xlow; gives position

y_position = (yhigh<<8) + ylow; //shift yhigh left and the add ylow; gives position

SPISSR = 1; //set back to high

delay_us(25);

}